Английский язык

Технический перевод (монтажные работы)

| Оригинал (англ) | Перевод (русс) |
| --- | --- |
| Instructions for correct refrigerant installation _ The execution of refrigerant lines for connection with the evaporator section must be properly performed by the client in compliance with current laws. It is especially advisable to refer to standard EN 378-2 and subsequent modifications. _ The refrigerant lines must be in copper piping for refrigeration systems, type EN 12735-1-2, electrolytic, mild copper, degreased, and de-oxidised. Make sure there are no impurities or moisture inside the pipes as this is extremely harmful for the refrigerant circuit. _ It is advisable to insulate the liquid line in cases where the external temperature (because of solar radiation) is higher than the temperature of the liquid itself. _ It is advisable to insulate the gas line to avoid burns from accidental contact, or to avoid heating indoor areas. _ Properly size the refrigerant lines so as to obtain limited leaks and a speed of the refrigerant liquid that guarantees carrying of the oil. _ The diameters of the inlet/outlet pipes of the remote condensers are shown in the table of dimensional characteristics on page 7. The table below shows the recommended dimensions for the refrigerant connection between the remote condenser and the evaporator unit, based on equivalent length. _ It is advisable to install, between the evaporator and the remote condenser, an anti-vibration device (1) and a silencer (2) so as to reduce the transmission of noise along the ducting (fig. 10). _ Horizontal stretches of pipe should end up sloping gently towards the low point (in the sense of gas flow), to favour the flow of oil. The slope should be between 0.5% and 1%. | Инструкция по установке хладагента _Подведение трубопровода хладагента к испарителю должно осуществляться клиентом строго в соответствии с действующими нормами. Рекомендуется обратиться к стандарту EN 378-2 и его последующим редакциям. _Трубопровод хладагента должен быть изготовлен из электролитической, мягкой, обезжиренной и раскисленной меди для систем охлаждения марки EN 12735-1-2. Убедитесь, что внутри труб нет влаги и примесей, так это крайне вредно для циркуляции хладагента. _Рекомендуется изолировать жидкостный трубопровод в случаях, когда температура снаружи (из-за солнечных лучей) выше температуры самой жидкости. _Рекомендуется изолировать газовый трубопровод в целях избежания перегрева помещения и ожогов при случайном контакте. _Правильно подберите размеры трубопровода хладагента так, чтобы избежать утечек и обеспечить свободную проходимость холодильной жидкости, необходимую для переноса масла. _ Диаметры впускных и выпускных труб выносных конденсаторов приведены в таблице размерных характеристик на стр. 7. В таблице ниже приведены рекомендуемые размеры для соединения выносного конденсатора и испарителя, основанные на эквивалентной длине. _Между испарителем и выносным конденсатором рекомендуется установить антивибрационное устройство (1) и глушитель (2) для снижения передачи шума по воздуховоду (рис.10). _Горизонтальные участки трубы должны заканчиваться наклонно к низшей точке (по направлению газового потока), чтобы способствовать проходу масла. Наклон может составлять от 0,5% до 1%. |